Faster, reliable card matching

Improvement options for scanapp and PokeSingle

7 October 2026

Your own measurements show where the time goes, and the two biggest fixes improve speed and reliability together.

# Where a scan spends its time

From five staging scans in data/latency/staging-speed030-after (small sample):

| Stage | Median |
| --- | --- |
| Server total | 4.4 s |
| OCR (title strip + footer strip) | 2.6 s |
| DINOv2 embeddings, 6 per scan | 1.0 s |
| Artwork embedding | 0.26 s |
| Frame proposals | 0.15 s |
| Catalogue search | 0.05 s |

Upload-to-result on the phone was about 5–9 s in the other runs. None of those five scans ended as matched; all were uncertain or printing_ambiguous.

# Highest-impact changes

- Straighten the card on the phone. This is the biggest win for both goals.

  - Speed: five of the six embeddings per scan are alternative crops the server tries because it doesn’t trust the frame. A rectified upload with skip_detect=true skips them, saving roughly 0.8–1 s (an estimate).

  - Reliability: an undetected or inferred frame can never be matched, and the title and footer OCR strips only line up on a properly cropped card.

  - How: opencv_dart doesn’t build for you, so use a small platform channel: Vision rectangle detection on iOS and OpenCV’s Android SDK on Android.

- Stop running slab grading on every scan.

  - Grading OCR ran alongside every one of those scans for a median 4.4 s.

  - In most of them it had not finished when the card result was ready, so it was cancelled and the work thrown away.

  - Meanwhile it competes with card OCR for CPU. Run it only when a slab is likely (frame shape, or a “graded” toggle in the app).

  - A clear drop in OCR time is expected, but it needs measuring.

- Skip footer OCR when it cannot change the answer. The collector number only matters when same-art reprints exist. If the top card has no such siblings and the visual score and gap are already strong, read the title only. This is a policy change, so replay it on your frozen panel before enabling.

- Merge stream_results into main. Your staging test showed a provisional result at 1.7 s against 6.9 s for the final. The app already supports it, so it is the cheapest perceived-speed gain.

- Speed up photo preparation on the phone.

  - The pure-Dart decode, crop, resize and encode took about 3 s for a 12 MP photo in your host benchmark; it has not been measured on a device.

  - A native codec (for example flutter_image_compress), or capturing at 4K instead of ResolutionPreset.max, should cut most of that.

  - Also test JPEG quality 85 instead of 95 for a smaller upload.

# Reliability beyond that

- Label your scans and calibrate. datasets/review has 61 scans and zero confirmed labels, so the thresholds (0.78, 0.70, 0.04) and rerank weights are hand-set. User confirmations already reach the server; turn them into a labelled set and tune against it. This is the safe way to move scans from uncertain to matched.

- Use ML Kit as a gate, not a replacement. Your iPhone 15 test: 43 ms against 1.6 s for server OCR, but collector-number recall of 54% against 87%. That is good enough to hold the shutter until the name and number are readable, and to send as extra hints. The server needs the ocr_text and collector_hint fields added, treated as candidate proposals only.

- Fine-tune the embedder (longer term). DINOv2 is used as published. Training it on catalogue images with glare, sleeve and perspective augmentation is the largest accuracy lever left, but it means training infrastructure and re-embedding the catalogue.

# Not worth doing

A vector database or FAISS: the search is 50 ms of a 4.4 s scan.

# Suggested order

Start with 2 and 4, since they are small server-side changes, then 1.